| Model | 450FG/1 |
| --- | --- |
| Max. drilling capacity | 45mm |
| Max. end milling capacity | 32mm |
| Max. face milling capacity | 80mm |
| Max. boring capacity | 70mm |
| Step of spindle speed | 6 |
| Spindle speed | 80-1250rpm |
| Spindle taper | MT4 or R8 |
| Spindle stroke | 130mm |
| Spindle auto feeding range | 0.06-0.30mm/r |
| Spindle nose to table | 440mm |
| Spindle axis to column | 273mm |
| Working table size | 800x240mm |
| T slot | 3x14mm |
| Y axis travel of worktable | 500mm |
| X axis travel of worktable | 175mm |
| Vertical swivel of spindle box | ±90° |
| Motor power | 1.5kw |
| Net weight | 340kg |
| Packing size | 770x880x1160mm |

| Items Description | Quantity |
| --- | --- |
| Drilling chuck with taper shank | 1 set |
| Arbor | 1 set |
| Drawbar | 1 set |
| Safety cover | 1 set |
| Nessary tools & tool kit | Numbers |